Club name: Denbigh GC  Site Name: Lleweni Parc

| Site Hazard | Mitigation / training or skill required |
| --- | --- |
| Narrow runway | Pilots must be able to run straight after landing, especially in crosswind conditions |
| Restricted landing area | Pilots require the skill to land accurately and stop at the far end of the runway |
| Challenging, non – linear ridge to soar | Pilot’s who haven’t soared our ridge previously should speak with the duty instructor before ridge soaring here. Experienced ridge soaring pilots may only need a briefing. |
| Turbulence on approach | When our ridge is working, the approach to the Westerly runway is challenging. Pilots must be aware they may have to land with very strong wind gradients and turbulence. Check flights available on request. Minimum half airbrake approach with higher than normal approach speeds are recommended in a strong headwind. |
| Public footpaths | Pilots should be aware that there are popular public footpaths on the ridge. Pilots should fly with sufficient margins such that members of the public can be avoided should they be using the footpaths. Pilots should never fly close to or overhead members of the public. |
| Wave soaring | Our site experiences common wave lift conditions to extreme altitudes. Pilots should receive training in navigation, cloud descent techniques, and the use of oxygen. Optionally, pilots may wish to receive training in cloud flying |
| Remote land outs | Our club is surrounded by very remote areas. Pilots should ensure that they are dressed appropriately and that they carry appropriate survival equipment and a method of electronic location. |